Техническая спецификация и архитектура

Проект «Третья рука»: C4-like схемы, ТЗ, модульная архитектура, edge/server взаимодействие

| Статус | Draft v0.7 |
| --- | --- |
| Дата | 20.03.2026 |
| Цель документа | Зафиксировать технический контур для Stage 1 -> Stage 3 и связать его с reuse-подходом OrcaHand |

1. Архитектурный принцип

Базовое решение: вся real-time логика движения и safety должна исполняться локально. Сервер нужен для телеметрии, хранения артефактов, обучения моделей, экспериментов и доставки конфигов/моделей. Это не догма ради красоты, а вывод из характера задачи: handover и shared control чувствительны к latency, user override и предсказуемости [S9][S10]. Решение v0.4: Stage 1 ориентирован на настольный dock как primary deployment target; body mount рассматривается как downstream packaging choice, проверяемый после bench validation.

Решение: Edge-first architecture. Сервер — вторичный слой. Он не должен быть single point of failure для запуска handover-сценария.

2. C4-like диаграммы

Context view: кто с кем взаимодействует и где проходит граница edge/server.

Container view: логические контейнеры внутри системы.

Component flow: путь сигнала от сенсоров до выдачи объекта.

3. Логические слои системы

| Слой | Назначение | Комментарий |
| --- | --- | --- |
| Mechanical layer | arm core, base, wrist, gripper, cable routing, printed shells | Должен жить независимо от конкретной inference-модели |
| Sensing layer | sEMG, IMU, button, optional camera | Stage 1 может стартовать без vision |
| Control layer | preprocess, intent, shared control, motion planner, safety | Главный edge-контур |
| Device layer | servo bus client, calibration, config, health | Здесь OrcaHand дает наибольшую пользу |
| Experiment layer | logging, replay, datasets, metrics | Нужен уже со Stage 1, иначе Stage 2 будет строиться вслепую |
| Server layer | artifact registry, dashboards, model versions, sync | Не участвует в обязательном локальном цикле управления |

4. Reuse map from OrcaHand

4.1 Direct adaptation candidates

• Device API: FastAPI + uvicorn + базовые команды connect/disconnect/status/enable torque/set joint positions/calibrate [S3][S4].

• Model folders и YAML-конфиги: отдельные папки моделей, где живут config.yaml и calibration.yaml [S2][S5].

• Hardware abstraction: motor_client + конкретные clients для Dynamixel и Feetech; mock client для bench tests [S2].

• CLI-паттерн: scripts/ для calibrate, neutral, record/replay, diagnostics [S2].

• URDF/MJCF-дисциплина: отдельный репозиторий описаний/сцен для симуляции [S6].

4.2 What must be redesigned

• Own arm model schema: joints arm_base / shoulder / elbow / wrist_yaw / wrist_pitch / gripper вместо finger joints.

• New calibration sequence for arm kinematics and dock alignment.

• Safety supervisor поверх device layer: зоны near-body, max current, max speed, collision fallback.

• Task/session context store: объект доступен где, какая фаза задачи, confidence, autonomy level.

• Handover policy и planner: этого в orca_core нет, это ядро нового продукта.

5. Initial technical assignment (ТЗ v0.1)

5.1 Scope for Stage 1

• Собрать arm core не менее чем с 3 активными степенями свободы плюс простой gripper/end-effector.

• Реализовать dock base как primary deployment target для bench режима, отладки и repeatability tests.

• Реализовать ручной trigger handover: кнопка либо gesture/IMU-pattern; на Stage 1 триггер должен работать в настольной конфигурации без body-worn wiring.

• Интегрировать как минимум 1 sEMG канал и 1-2 IMU для логирования и пилотного intent classification.

• Поддержать 3-5 заранее известных объектов в фиксированной кассете.

• Сделать safe pose, return-to-home, E-stop и watchdog.

• Записывать сессии в локальное хранилище с возможностью replay.

• Завершить Stage 1 отдельным geometry/mount study по готовому настольному core-модулю: масса, CoG, envelope, reach, interference, candidate mounts.

5.2 Scope for Stage 2

• Добавить readiness model / phase model пользователя.

• Перейти на shared control: trigger becomes optional, not mandatory.

• Сделать body-mounted wearable alpha с ограниченным payload.

• Ввести profile-based configs per user/per task.

• Поддержать offline labeling и re-training pipeline.

5.3 Scope for Stage 3

• Quick-swap dock/wearable base.

• Model registry и конфигурационный сервер.

• Сменные end-effectors и tool cartridges.

• Режимы автономии по профилю задачи.

• Накопление собственного dataset и benchmark suite.

6. Предлагаемая software architecture

| Контейнер | Где работает | Функция | Исходная база |
| --- | --- | --- | --- |
| sensor_ingest | Edge SBC | Сбор sEMG/IMU/button/camera timestamps | Новое |
| signal_preprocess | Edge SBC | Filtering, normalization, windows, sync | Новое |
| intent_service | Edge SBC | Classification/regression for trigger/readiness/object request | Новое |
| shared_control_core | Edge SBC | Combines user intent + policy + manual override | Новое, но опирается на [S10] |
| motion_planner | Edge SBC | Pose selection, trajectories, velocities | Новое |
| safety_supervisor | Edge SBC | Limits, watchdog, near-body rules, current/temp fences | Новое |
| device_service | Edge SBC | HTTP API + hardware access + calibration CLI | Reuse pattern from OrcaHand [S3][S4] |
| telemetry_client | Edge SBC | Upload logs/config snapshots to server | Новое |
| experiment_server | Remote/local PC | Artifacts, models, metrics, dashboards | Новое |

7. API sketch

Ниже — рабочая форма будущего device API. Она не копирует OrcaHand 1:1, но наследует его логику: состояние устройства, явные команды, отдельный config endpoint, простая калибровка и stateless HTTP-слой поверх локального control core [S4].

| Method | Endpoint | Назначение | Примечание |
| --- | --- | --- | --- |
| POST | /config | Выбор модели/base/task profile | YAML/JSON config snapshot |
| POST | /connect | Подключить arm module | Локальный вызов |
| POST | /disconnect | Отключить модуль | Сначала torque off |
| GET | /status | Состояние edge loop, calibration, faults | Health-check |
| POST | /safety/estop | Аварийная остановка | Highest priority |
| POST | /mode | dock / wearable / replay / safe | Mode switch |
| POST | /handover/request | Явный запрос на подачу объекта | Stage 1 core |
| POST | /intent/debug | Подача тестового намерения/меток | Для bench tests |
| GET | /sensors/live | sEMG/IMU snapshot | Debug/inspection |
| POST | /calibrate | Калибровка arm/base/sensors | May take time |
| POST | /joints/position | Низкоуровневое движение для сервисных сценариев | Не user-facing |
| POST | /session/start | Старт журналирования | Links all streams |
| POST | /session/stop | Стоп журналирования и упаковка артефактов | Creates replay bundle |

8. Data and config model

• arm_model.yaml — joints, actuator ids, limits, neutral pose, base type, payload profile.

• sensor_profile.yaml — sEMG placement, IMU ids, sample rates, filter params.

• task_profile.yaml — object set, handover zone, orientation presets, autonomy level.

• safety_profile.yaml — current limits, speed limits, workspace fences, body exclusion zones.

• session bundle — timestamps, raw sensors, inferred intent, executed trajectory, faults, user notes.

Почему именно так: Если конфиги не вынести отдельно и не версионировать, Stage 2 неизбежно превратится в набор не повторяемых «магических настроек». OrcaHand полезен как раз тем, что уже показывает дисциплину model/config folders [S2][S5].

9. Latency, reliability, safety assumptions

| Параметр | Draft target | Комментарий |
| --- | --- | --- |
| Intent loop frequency | 50-100 Hz | Достаточно для Stage 1/2 пилота; уточнить после выбора железа |
| Device command loop | 50-200 Hz | Зависит от actuator bus |
| Server dependency | 0 для base mode | Сервер optional при реальном handover |
| Manual override reaction | Максимально быстро, локально | Не должен ждать сеть/облако |
| Fail-safe on sensor loss | Fallback to safe pose / stop | Особенно критично для sEMG noise |
| On-body speed policy | Conservative by default | Body-mode медленнее dock-mode |

10. Test strategy

• Bench tests: dry run without object, then with dummy objects, then with target objects.

• Replay tests: повторное воспроизведение журналов и intent streams.

• Fault injection: sensor dropout, stale command, overcurrent, disconnect, bad confidence.

• Human factors: скорость, комфорт, perceived intrusion, reachability, trust.

• Wearable tests: сначала без выдачи в руку, потом с пустым объектом, потом с реальным lightweight object.

11. Open technical decisions

| ID | Решение | Текущий статус |
| --- | --- | --- |
| D-01 | Actuator stack | Feetech-first для бюджетного PoC или Dynamixel-first для SDK/robustness? |
| D-02 | Base mount | waist/hip vs torso/shoulder as primary wearable mount |
| D-03 | Vision timing | camera in Stage 1 or Stage 2 |
| D-04 | End-effector type | simple gripper vs magnetic/tool-specific holder |
| D-05 | Power | external PSU first vs body battery from alpha |
| D-06 | Compute | Pi 5 enough for Stage 1, or local mini-PC needed sooner |

Источники

[S1] ETH Zurich Soft Robotics Lab: ORCA hand overview, 17-DoF tendon-driven hand, material cost below 2,000 CHF, assembly <8h, reliability claims — https://srl.ethz.ch/orcahand.html

[S2] GitHub: orcahand/orca_core repository overview and scripts for tension/calibration/neutral/test — https://github.com/orcahand/orca_core

[S3] raw pyproject.toml: orca_core depends on dynamixel-sdk, FastAPI, uvicorn, requests/httpx, numpy and targets Python >= 3.10 — https://raw.githubusercontent.com/orcahand/orca_core/main/pyproject.toml

[S4] raw api.py: device API with /connect, /status, /torque, /joints/position, /calibrate endpoints — https://raw.githubusercontent.com/orcahand/orca_core/main/orca_core/api/api.py

[S5] raw config.yaml: YAML-based motor/joint mapping, control_mode, calibration sequence — https://raw.githubusercontent.com/orcahand/orca_core/main/orca_core/models/orcahand_v1_right/config.yaml

[S6] GitHub: orcahand_description repository with URDF/MJCF and MuJoCo scene — https://github.com/orcahand/orcahand_description

[S7] CORDIS HARIA project: wearable and grounded supernumerary limbs, wearable interfaces, shared AI/user control trade-off — https://cordis.europa.eu/project/id/101070292

[S8] Frontiers 2024 review: wearable augmentations, exoskeletons and supernumerary limbs, non-invasive EEG/sEMG intent interfaces — https://www.frontiersin.org/journals/neurorobotics/articles/10.3389/fnbot.2024.1443010/full

[S9] ScienceDirect 2024 review: human-robot object handover as a core capability in collaboration — https://www.sciencedirect.com/science/article/pii/S2667379724000032

[S10] PubMed 2025: shared control for SRL improves task success and user experience vs fully autonomous or voluntary-only control — https://pubmed.ncbi.nlm.nih.gov/39996972/

[S11] ETH 3HANDS dataset: naturalistic handovers for hip-mounted SRL, intimate workspace and primary-task user — https://www.research-collection.ethz.ch/entities/publication/6aa8d0e4-f25f-41d4-9bc9-176b85b9df57

[S12] MDPI 2025 review: sEMG intention recognition is central but difficult; fusion with other signals is a major direction — https://www.mdpi.com/1424-8220/25/8/2448

[S13] ScienceDirect 2025 review: sEMG-IMU fusion improves accuracy and reliability for upper-limb movement recognition — https://www.sciencedirect.com/science/article/pii/S1566253525004956

[S14] Eckstein: Feetech STS3215 current retail price and specs — https://eckstein-shop.de/Feetech-STS3215-74V-19kgcm-plastic-case-metal-gear-magnetic-code-dual-shaft-TTL-serial-servo

[S15] BerryBase: Waveshare ST/SC serial bus servo adapter board current retail price — https://www.berrybase.de/en/waveshare-serial-bus-servo-adapter-board-253-st-sc-series-controller-for-robot-arm-hexapod

[S16] BerryBase: Raspberry Pi 5 8GB current retail price — https://www.berrybase.de/en/raspberry-pi-5-8gb-ram

[S17] Antratek: MyoWare 2 sensor current retail price — https://www.antratek.de/myoware-2-0-muscle-sensor-basic-kit

[S18] BerryBase / DigiKey: BNO085 breakout and chip current retail price — https://www.berrybase.de/en/adafruit-9-dof-orientation-imu-fusion-breakout-bno085-bno080

[S19] MyBotShop / pib.Shop / Luxonis: OAK-D Lite current retail price — https://www.mybotshop.de/Luxonis-DepthAI-OAK-D-Lite_1

[S20] MyBotShop/Reichelt/Funduino: ROBOTIS U2D2 current retail price — https://www.mybotshop.de/ROBOTIS-DYNAMIXEL-U2D2_1

[S21] ROBOTIS XM430-W350-T official specs and price — https://en.robotis.com/shop_en/item.php?it_id=902-0124-000

[S22] ROBOTIS / MYBOTSHOP: XL330-M288-T official specs and retail price — https://en.robotis.com/shop_en/item.php?it_id=902-0163-000

# v0.2 — Архитектурные уточнения после выбора payload и object set

Ниже зафиксированы изменения, которые должны повлиять на механику, safety envelope и server/edge границу.

System class for Stage 1: rigid-object wearable-first handover assistant with docked bench mode.

Primary Stage 1 objects: mug, flashlight, ball. Secondary objects: marker, screwdriver.

Deformable objects (towel, cord) are excluded from Stage 1 and Stage 1.5 architecture assumptions.

Vision is added in Stage 1.5 as a non-safety-critical perception module; Stage 1 remains trigger-driven without scene reasoning.

Quick-change tool-side is postponed, but mechanical/electrical interface reservation is required in arm core.

## Updated safety envelope

Nominal payload mode: up to 300 g object mass with conservative acceleration.

Stretch mode: up to 500 g only at reduced velocity and short reach envelope.

Mug scenario is approved only as empty mug or closed-lid mug in Stage 1 unless spill-safe tests are passed.

No cloth, rope, towel or dangling cable handling in Stage 1 baseline.

Wearable mode must allow emergency stop, manual override and passive safe pose fallback.

## Mounting trade study to be performed before hardware freeze

| Вариант | Плюсы | Риски | Статус |
| --- | --- | --- | --- |
| Пояс/бедро | ниже центр масс, проще надеть | может мешать походке и рабочей зоне | исследовать |
| Рюкзак/спина | лучше для батареи и heavier core | сложнее кинематика и доступ к зоне подачи | исследовать |
| Плечо/торс | короче путь до руки пользователя | выше burden и выше требования к безопасности рядом с шеей/лицом | осторожно |

## Server interaction refinement

Server still stays out of real-time control loop.

Stage 1.5 vision inferences may run locally on edge and sync metadata/models to server asynchronously.

Object profiles should be represented as typed recipes: mass range, preferred grasp pose, delivery orientation, safe speed cap.

Future quick-change end-effectors should be exposed as capability profiles, not hard-coded device branches.

# v0.3 — Технические уточнения по Stage 1 / 2 / 3

Tool-side strategy: Stage 1 использует один фиксированный utility end-effector, но arm core обязан иметь заранее предусмотренный mounting pattern и reserve space под future electrical contacts / quick-release. Stage 2 вводит manual swap end-effectors. Stage 3 вводит полноценный quick-change tool-side.

Stage 1 handover default: режим A — object is brought to pickup zone and held stable. Stage 1 stretch: режим C — object moved to fixed handover waypoint near dominant hand. Режим B откладывается до Stage 2, когда появятся readiness estimation, lower-speed final approach и более тонкий release policy.

Рабочий reach для v1: 30–35 см от базовой точки выдачи. Дальнейшее увеличение только после валидации torque budget и burden near body.

Cup handling policy: first-pass object model = empty cup; slightly-filled cup allowed only as stretch test with reduced speed profile and spill-safe posture assumptions.

Proto-industrial design: корпуса и крышки должны быть аккуратными уже в Stage 1, но cosmetic shells не должны мешать обслуживанию, доступу к проводке и ревизии приводов.

## Обновление ТЗ Stage 1

Обязательная дальность handover: 30–35 см.

Обязательный default сценарий: поднести и удерживать объект в pickup zone.

Желательный stretch сценарий: вывести объект в фиксированный waypoint рядом с правой рукой.

Пилотные объекты: пустая кружка, фонарик, мячик.

В Stage 1 не требуется полноценный quick-change; требуется лишь стандартный интерфейс крепления будущих end-effectors.

## Изменения v0.4

Primary deployment for Stage 1 = desktop dock. Edge stack, safety supervisor and planner отлаживаются без телесного mount.

Mount architecture на Stage 1 держится как interface contract: arm core должен иметь точки крепления и кабельный запас, но финальная wearable base не фиксируется.

Mount study становится отдельным technical gate перед Stage 2.

10. Architectural delta v0.5 — split powered-base strategy

Новая фиксация архитектуры: arm core переиспользуется между режимами, но base modules не унифицируются как одинаковый корпус. Унифицируется интерфейс между arm core и base module: mechanical lock, power bus, comms bus, emergency line.

Common interface contract: arm-core flange + 1 power connector + 1 data connector + fast detach lock.

Desktop Dock Module responsibilities: fixed bench mount, stable footprint, optional external PSU or dock battery, compute host, logging and calibration station.

Wearable Mount Module responsibilities: body-safe mounting frame, cable routing, soft interfaces to body, optional separated battery/computer pack, emergency quick release.

Low-level safety remains on arm/edge controller; body-mode must not depend on server or on dock-resident software for stop behavior.

Причина такого split: mounting position, link lengths и arm burden критически меняют usable workspace в wearable robotics, а тяжелые батареи и масса в основании полезны для stability на стационарной платформе, но не переносятся напрямую на тело без ухудшения ergonomics и acceptance.

12. Server interface, uplink and programming path

Northbound interface between base module and server is intentionally separated from southbound motion/control interfaces. The server is not allowed to bypass local safety and talk directly to actuator buses.

12.1 Recommended connectivity model

Southbound (arm core ↔ base module): platform contract from v1.0 — 24 V main rail, CAN-based external module bus, hardwired NC safety loop, presence/interlock lines.

Local edge compute/gateway on desktop dock: Raspberry Pi 5 class SBC. It already provides Gigabit Ethernet, dual-band 802.11ac Wi‑Fi and Bluetooth/BLE, so no отдельный Wi‑Fi module не обязателен для Stage 1 при выборе Pi 5 [S33].

Northbound (base module ↔ server): два режима. (a) Bench/service mode — Ethernet or direct local connection for programming and diagnostics. (b) Wireless ops mode — Wi‑Fi uplink for telemetry, dashboard, config sync and non-critical “brain” services.

Optional wearable successor: CM5-class module with optional Wi‑Fi/Bluetooth [S34][S35]. Это полезно, если в Stage 2/3 desktop Pi 5 станет слишком большим для body-side packaging.

Device API northbound: HTTPS/REST for configs, jobs, artifacts and logs ingestion; WebSocket stream for live status/events/operator UI. This mirrors the existing FastAPI-friendly software direction and keeps implementation complexity low [S36].

Local persistence: ring buffer + append-only session bundles on edge. Server outages must not break tests; sync is eventually consistent.

Programming path: default programming over Ethernet/USB service access in Stage 1; OTA package pull over Wi‑Fi allowed later, but only after signed/versioned update flow is in place.

12.2 Practical recommendation by stage

Stage 1: use Ethernet as primary dev/debug path, keep Wi‑Fi enabled for convenience, dashboards and log upload.

Stage 1.5: add Wi‑Fi-based telemetry and live operator page once base handover loop is stable.

Stage 2: if wearable alpha is built, keep local autonomy on-device and treat Wi‑Fi as supervisory link only.

Stage 3: move toward formal config/model registry and remote fleet-style management, still without putting real-time motion in the server path.

12.3 Sources added for v0.6

[S33] Raspberry Pi 5 official product page: Gigabit Ethernet, dual-band 802.11ac Wi‑Fi, Bluetooth 5.0/BLE — https://www.raspberrypi.com/products/raspberry-pi-5/

[S34] Raspberry Pi documentation: Raspberry Pi computer hardware connectivity summary — https://www.raspberrypi.com/documentation/computers/raspberry-pi.html

[S35] Raspberry Pi Compute Module 5 official page / docs: CM5 with wireless options — https://www.raspberrypi.com/products/compute-module-5/ ; https://www.raspberrypi.com/documentation/computers/compute-module.html

[S36] FastAPI WebSocket docs — https://fastapi.tiangolo.com/advanced/websockets/

13. Northbound Protocol Contract v1.0

Contract относится к связи base module / local edge compute с сервером и внешними приложениями. Он отделен от southbound motion/control interfaces и не должен участвовать в safety-critical real-time loop.

13.1 Chosen pattern

Mandatory baseline: HTTPS/REST + WebSocket.

Planned extension: MQTT telemetry/event bus from Stage 1.5/2+ when multi-client subscriptions or decoupled event processing become useful.

Reference deployment for Stage 1: FastAPI service on local edge compute; REST for jobs/config/artifacts, WebSocket for live telemetry/events/operator channel.

13.2 Message families

status - current mode, lifecycle state, arm present, latch state, network state, current job.

telemetry - sampled summaries from joints/sensors/intent/confidence, battery/power, temperatures, watchdog counters.

event - discrete domain events: homed, handover_started, object_ready, fault_raised, estop_pressed, sync_complete.

command/job - high-level operations only: start_session, stop_session, home, calibrate, handover_request, export_logs, apply_config.

artifact/config/model - versioned configs, model files, session bundles, replay files, calibration profiles.

fault - structured alarms with code, severity, source, timestamp, recommended action.

13.3 Hard boundaries

Server cannot send raw joint/motor commands into actuator bus.

Loss of northbound link must not prevent local stop, return-to-safe-pose or session completion policy.

Local queue/spool is mandatory for logs and events when uplink is absent or unstable.

Northbound auth/versioning may evolve, but message schema stability is required once Stage 1 API is frozen.

13.4 Stage mapping

Stage 1: REST + WebSocket only; MQTT optional and normally omitted for the fastest first result.

Stage 1.5: add basic vision and, if needed, MQTT topics for telemetry/events consumed by separate UI/logging services.

Stage 2: keep local autonomy on edge, treat northbound as supervisory/control plane; MQTT becomes justified if event fan-out appears.

Stage 3: formalize config/model registry, replay pipelines and remote orchestration while keeping real-time motion local.

9. Stage 1 API and message contract freeze (v0.8)

Базовый принцип. HTTP-слой документируется через OpenAPI, а event-driven слой WebSocket/MQTT — через AsyncAPI. Это разделяет request/response и message-driven части без дублирования схем.

9.1 Recommended JSON envelope

{

"schema_version": "1.0",

"msg_type": "telemetry | event | fault | ack",

"msg_name": "handover.completed",

"ts": "2026-03-21T18:42:10.123Z",

"source": "edge.desktop.alpha1",

"session_id": "sess_01",

"seq": 1842,

"payload": {}

}

9.2 Stage 1 WebSocket stream

| Channel | Direction | Payload families | Notes |
| --- | --- | --- | --- |
| /ws/stream | server -> client | status, telemetry, event, fault, ack | Single multiplexed stream; simpler reconnect |
| /ws/stream | client -> server | operator command envelopes (optional later) | Stage 1 may keep commands on REST only |

9.3 Stage 1 command semantics

| Command | Input | Synchronous response | Async side effect | Priority |
| --- | --- | --- | --- | --- |
| session.start | profile_id, operator, notes | 202 Accepted + command_id | event: session.started | must |
| arm.home | none / pose id | 202 Accepted + command_id | event: arm.homed / fault | must |
| arm.calibrate | calibration_mode | 202 Accepted + command_id | event: arm.calibrated / fault | must |
| handover.request | object_id, source_slot, present_mode | 202 Accepted + command_id | events: started, grasped, presented, completed | must |
| arm.safe-stop | reason | 202 Accepted + command_id | event: arm.safe_stopped | must |
| arm.reset-fault | fault scope | 200/202 | event: fault.cleared | must |

9.4 MQTT-compatible topic schema (reserved)

thirdarm/{device_id}/status

thirdarm/{device_id}/telemetry

thirdarm/{device_id}/events

thirdarm/{device_id}/faults

MQTT не обязателен в Stage 1. Схема резервируется заранее, чтобы позже подключить broker без переосмысления message families.

9.5 Implementation notes for FastAPI edge service

REST handlers должны возвращать command_id и не блокироваться на полный жизненный цикл handover.

WebSocket stream должен нести только нормализованные сообщения, а не сырые servo packets.

Replay/logging пишет ровно те же envelopes, что идут в /ws/stream, чтобы не плодить второй формат.

Внутренний southbound слой может использовать иной transport и иную частоту обновления; northbound должен отдавать агрегированную, стабильную картину.